Useful Circuits

Provided by TryEngineering - www.tryengineering.org

Lesson Focus

Demonstrate and discuss progression from simple circuits (second level) to parallel circuits (third level), the differences between series and parallel circuit design and functions and the use of parallel circuits in every day applications.

Lesson Outline

These circuit activities encourage students to test different circuit designs through the use of low voltage light bulbs, buzzers and switches.  Students work in teams to design the circuits, and then build examples of the different circuits using the selected equipment.  The groups can then compare results and discuss findings.

Age Levels

P6 to S2

Learning Outcomes

As a result of this activity, students should be able to:

- Design circuits that would be useful in a range of situations.

- Develop an understanding of the uses of series and parallel circuits.

- Discuss and develop skills in making and testing predictions and drawing conclusions.

- Develop teamwork skills.

Resources/Materials

- Teacher Resource Documents

- Student Worksheet

- Student Resource Sheets

Internet Connections

- TryEngineering (www.tryengineering.org)

- National Institute of Standards and Technology (NIST) (www.nist.gov)
Information about measurements and measurement uncertainty.

- ITEA Standards for Technological Literacy: Content for the Study of Technology
(www.iteaconnect.org/TAA)

- National Science Education Standards (www.nsta.org/standards)

Recommended Reading

- DK Eyewitness Series: Electricity (ISBN: 0751361321)

- Make Cool Gadgets for Your Room by Amy Pinchuk and Teco Rodriques  (ISBN: 1894379128)

- My World of Science: Conductors and Insulators by Angela Royston
(Heinemann Educational Books, ISBN: 0431137269)

Possible Literacy Activity

- Would replacing one light on a string of Christmas tree lights with a "blinking" light cause all the lights in the string to also blink?  Is this an example of a parallel or series circuit?  Why? Discuss your answer fully in a format of your choosing.

Possible Assessment Opportunities

- Literacy activity

- Student worksheet

- Self/peer assessment

- Teacher observation

Useful Circuits

For Teachers:

Progression through Curriculum for Excellence

Lesson focus:

The ideas in these lessons can span between primary and secondary, either through close links between sectors, or though a transition project as follows:

Second level (P5 to P7)

I have used a range of electrical components to help to make a variety of circuits for differing purposes. I can represent my circuit using symbols and describe the transfer of energy around the circuit.

SCN 2-09a

Third level (S1 to S3)

Having measured the current and voltage in series and parallel circuits, I can design a circuit to show the advantages of parallel circuits in an everyday application.

SCN 3-09a

Beyond third level, pupils can progress onto fourth level work as follows:

Through investigation, I understand the relationship between current, voltage and resistance. I can apply this knowledge to solve practical problems.

SCN 4-09a

Useful Circuits

Teacher Resources

 Materials

- Student Resource Sheets

- Student Worksheet

- Notebooks

- Pencils

- A set-up for each group of students, each consisting of:

  - Power supply (1.5V batteries and holders or 6V lab packs)

  - Three or more bulbs to suit the power supply (1.5V or 6V)

  - Switch

  - Wires/leads

  - Crocodile clips

  - Buzzer

  - Tin foil

  - Card

  - Paper fasteners (for quiz game)

  - Wire (for quiz game)

  - Red, amber and green painted bulbs (for traffic lights)

 Procedure

Explain the definitions of a series circuit with the class.  Use Student Reference Sheets for background information.  These may also be distributed as homework reading on the night prior to the activity.

Divide students into small groups of 3-4 students and distribute Student Worksheet and the set-up (see materials above) to each group.

Have each student group to design a useful circuit as follows:  a doorbell for the hearing impaired, a pressure sensitive burglar alarm, an alarmed drawer and an electronic quiz game

Have each student group test their predictions using their circuits, and compare their results to their predictions.

Student group should then try to design and construct 2 parallel circuits as follows:  model house and traffic lights.

Bring the student groups together to discuss their findings.

 Time Needed

4 to 5 hours

 Tips

- Teachers may consider distributing the student resource sheets as reading material/homework for the night before the class activity takes place.

- Encourage students to compare all the circuits built by different groups.

Useful Circuits

Student Resource:

What is a Simple Circuit?

 Simple Circuit

A simple circuit consists of three elements: a source of electricity (battery or lab pack), a path through which electricity flows (wire) and a bulb to show that the electricity is operating correctly.  The illustration below shows a simple circuit with a power supply, two wires, and a low voltage light bulb.  The flow of electricity is caused by the electrons on the negative end of the power supply flowing toward the positive end.  When the circuit is complete, electrons flow from the negative end through the wires, then through the bulb (lighting it up), and finally back to the positive end - in a continual flow.

 Diagram of a Simple Circuit

The following are circuit diagrams of the series circuit showing the symbols for the power supply, switch, and a bulb.

Useful Circuits

Student Resource:

What are Series and Parallel Circuits?

Series and parallel describes two different types of circuit arrangements. Each arrangement provides a different way for electricity to flow throughout a circuit.

 Series Circuits

In a series circuit, electricity has only one path on which to travel.  Electricity flows from the battery to each bulb, one at a time, in the order they are wired to the circuit. In this case, because the electricity can only flow in one path, if one of the bulbs blew out, the other bulb would not be able to light up because the flow of electric current would have been interrupted.  In the same way, if one bulb was unscrewed, the current flow to both bulbs would be interrupted.

 Parallel Circuits

In a parallel circuit, electricity has more than one path on which to travel.  In this case, because the electricity can flow in more than one path, if one of the bulbs blew, the other bulb would still be able to light up because the flow of electricity to the broken bulb would not stop the flow of electricity to the good bulb.  In the same way, if one bulb were unscrewed, it would not prevent the other bulb from lighting up.

Series and Parallel Circuits

Student Worksheet:

- Instructions

You are a security engineer!  You need to design a circuit where an alarm will sound if an intruder steps on it.  Then design a circuit where an alarm will sound a drawer is opened.

You are a broadcast engineer.  Design a circuit using circuit diagram symbols and correct answers, where the correct answer will light a bulb or sound a buzzer.

You are an electrical engineer!  You need to design a circuit where you can control two bulbs separately, with a switch, so that both bulbs are equally bright, and could be located in different rooms of a house.

Draw your own circuit diagram below showing how your circuit should be connected:

You are an civil engineer!  You need to design a traffic light circuit where the red, amber and green bulbs can be switched on and off separately, but the red and amber bulbs can also light together.

Draw your own circuit diagram below showing how your circuit should be connected:

Useful Circuits

Student Worksheet (continued):

 Group Predictions

After you have constructed your circuits, make some predictions on the following as a group:

1.  Do you think Christmas tree lights are an example of bulbs in series or in parallel?

2.  If you remove a bulb in your series circuit, with the other bulb(s) still light?  Explain why:

3.   If you remove a bulb in a parallel circuit, with the other bulb(s) still light?  Explain why:

 Test and Results

Test your predictions for questions 2, 3 and 4 above.  Now answer the questions below:

1.  Were your predictions about the brightness of the bulbs correct?  If not, what was different from what your group expected?

2.  Were your predications about the removal of a bulb from the series and parallel circuits correct?  If not, what happened that was different from what your group expected?